# Date:

TASK 7: Deutsch-Jozsa for 2-qubits

Aim: To implement and demonstrate the Deutsch-Jozsa algorithm for 2-qubit oracles, distinguishing between constant and balanced functions using quantum computation.

# 1 Mathematical Model of the Deutsch-Jozsa Algorithm for 2 Qubits

Given a function f:{00,01,10,11}→{0,1} the Deutsch-Jozsa algorithm determines whether f is constant (same output for all inputs) or balanced (outputs 0 for half the inputs, 1 for the other half), using only one quantum query. The following key steps in the quantum state evolution.

# Problem Setup

You  have  an  unknown  Boolean  function f:{0,1}n→{0,1}  promised  to  be either constant (same output for all inputs) or balanced (outputs 0 on exactly half the inputs, and 1 on the other half).

# Initial State

Prepare n+1 qubits: the first n in ∣0⟩⊗n and one ancilla in ∣1⟩

# Apply Hadamard Gates

Apply Hadamard gates to all n+1 qubits, creating a superposition.

# Query the Oracle Operation Uf

The oracle maps

Applying it imparts a phase

# Apply Hadamard on Input Qubits

- Measurement

  - If all measured bits are 0 (or ∣0⟩⊗n), the function f is constant.

  - Otherwise, f is balanced.

# 2 Algorithm - Deutsch-Jozsa for 2-qubits

- Initialize qubits |00⟩|1⟩

- Apply Hadamard to all 3 qubits

- Apply the Oracle Uf : Use a controlled operation based on the function f(x)

- Apply Hadamard gates to first 2 qubits

- Measure first 2 qubits

  - Measure input first 2 qubits.

  - Outcome |00⟩ occurs with probability 1 if f is constant.

  - Any other outcome means f is balanced.

# 3 Program

!pip install pennylane qiskit qiskit-aer

import pennylane as qml

from pennylane import numpy as np

import matplotlib.pyplot as plt

from qiskit import QuantumCircuit, transpile

from qiskit_aer import Aer

from qiskit.visualization import plot_histogram

import numpy as np

print("MATHEMATICAL MODEL")

print("=" * 50)

print("For function f: {00, 01, 10, 11} → {0,1}:")

print("- Constant: f(x) = 0 or 1 for all inputs")

print("- Balanced: f(x) = 0 for half inputs, 1 for other half")

print("\nQuantum State Evolution:")

print("1. |ψ₀⟩ = |00⟩|1⟩")

print("2. |ψ₁⟩ = H⊗³|ψ₀⟩ = ½∑|x⟩(|0⟩-|1⟩)/√2")

print("3. |ψ₂⟩ = U_f|ψ₁⟩ = ½∑(-1)^f(x)|x⟩(|0⟩-|1⟩)/√2")

print("4. |ψ₃⟩ = H⊗²|ψ₂⟩")

print("5. Measure: if |00⟩ → constant, else → balanced")

oracle_types = ['constant_zero', 'constant_one', 'balanced_x0',

'balanced_x1', 'balanced_xor', 'balanced_and']

def classical_truth_table(oracle_type):

if oracle_type == 'constant_zero':

return {'00': 0, '01': 0, '10': 0, '11': 0}

elif oracle_type == 'constant_one':

return {'00': 1, '01': 1, '10': 1, '11': 1}

elif oracle_type == 'balanced_x0':

return {'00': 0, '01': 0, '10': 1, '11': 1}

elif oracle_type == 'balanced_x1':

return {'00': 0, '01': 1, '10': 0, '11': 1}

elif oracle_type == 'balanced_xor':

return {'00': 0, '01': 1, '10': 1, '11': 0}

elif oracle_type == 'balanced_and':

return {'00': 0, '01': 0, '10': 0, '11': 1}

def constant_zero_oracle(): pass

def constant_one_oracle(): qml.PauliZ(wires=2)

def balanced_x0_oracle(): qml.CNOT(wires=[0, 2])

def balanced_x1_oracle(): qml.CNOT(wires=[1, 2])

def balanced_xor_oracle():

qml.CNOT(wires=[0, 2])

qml.CNOT(wires=[1, 2])

def balanced_and_oracle(): qml.Toffoli(wires=[0, 1, 2])

pennyLane_oracles = {

'constant_zero': constant_zero_oracle,

'constant_one': constant_one_oracle,

'balanced_x0': balanced_x0_oracle,

'balanced_x1': balanced_x1_oracle,

'balanced_xor': balanced_xor_oracle,

'balanced_and': balanced_and_oracle

}

dev = qml.device('default.qubit', wires=3, shots=1000)

def deutsch_jozsa_circuit(oracle_func):

qml.PauliX(wires=2)

for i in range(3):

qml.Hadamard(wires=i)

oracle_func()

qml.Hadamard(wires=0)

qml.Hadamard(wires=1)

return qml.probs(wires=[0, 1])

dj_qnode = qml.QNode(deutsch_jozsa_circuit, dev)

def create_dj_circuit_qiskit(oracle_type):

qc = QuantumCircuit(3, 2)

qc.x(2)

qc.h(0)

qc.h(1)

qc.h(2)

if oracle_type == 'constant_zero': pass

elif oracle_type == 'constant_one': qc.z(2)

elif oracle_type == 'balanced_x0': qc.cx(0, 2)

elif oracle_type == 'balanced_x1': qc.cx(1, 2)

elif oracle_type == 'balanced_xor':

qc.cx(0, 2)

qc.cx(1, 2)

elif oracle_type == 'balanced_and': qc.ccx(0, 1, 2)

qc.h(0)

qc.h(1)

qc.measure(0, 0)

qc.measure(1, 1)

return qc

def run_qiskit_circuit(oracle_type, shots=1000):

qc = create_dj_circuit_qiskit(oracle_type)

simulator = Aer.get_backend('qasm_simulator')

tqc = transpile(qc, simulator)

job = simulator.run(tqc, shots=shots)

result = job.result()

counts = result.get_counts()

return counts, qc

print("\n" + "="*50)

print("SAMPLE INPUT/OUTPUT FOR PENNYLANE AND QISKIT IMPLEMENTATIONS")

print("="*50)

results = []

for oracle_type in oracle_types:

print(f"\nTesting {oracle_type}:")

print(f"Classical truth table: {classical_truth_table(oracle_type)}")

oracle_func = pennyLane_oracles[oracle_type]

probs = dj_qnode(oracle_func)

is_constant_pl = probs[0] > 0.9

counts, circuit = run_qiskit_circuit(oracle_type)

zero_count = counts.get('00', 0)

is_constant_qk = zero_count / 1000 > 0.9

results.append({

'oracle': oracle_type,

'classical_type': 'Constant' if all(v == list(classical_truth_table(oracle_type).values())[0]

for v in classical_truth_table(oracle_type).values()) else 'Balanced',

'pennyLane_result': 'Constant' if is_constant_pl else 'Balanced',

'qiskit_result': 'Constant' if is_constant_qk else 'Balanced',

'pennyLane_p00': probs[0],

'qiskit_counts': counts

})

print(f"PennyLane: {results[-1]['pennyLane_result']} (P(|00⟩) = {probs[0]:.4f})")

print(f"Qiskit:    {results[-1]['qiskit_result']} (Counts: {counts})")

print("\n" + "="*50)

print("QUANTUM CIRCUIT EXAMPLES")

print("="*50)

example_oracles = ['constant_zero', 'balanced_x0', 'balanced_and']

for oracle_type in example_oracles:

print(f"\nCircuit for {oracle_type}:")

print("PennyLane:")

oracle_func = pennyLane_oracles[oracle_type]

print(qml.draw(dj_qnode)(oracle_func))

print("Qiskit:")

qc = create_dj_circuit_qiskit(oracle_type)

print(qc)

print("\n" + "="*50)

print("RESULTS VISUALIZATION")

print("="*50)

fig, axes = plt.subplots(2, 3, figsize=(15, 10))

axes = axes.flatten()

for i, result in enumerate(results):

states = ['00', '01', '10', '11']

pl_probs = [result['pennyLane_p00'], 0, 0, 0]

qk_counts = result['qiskit_counts']

qk_probs = [qk_counts.get(state, 0)/1000 for state in states]

x = np.arange(len(states))

width = 0.35

axes[i].bar(x - width/2, pl_probs, width, label='PennyLane', alpha=0.7, color='green')

axes[i].bar(x + width/2, qk_probs, width, label='Qiskit', alpha=0.7, color='blue')

axes[i].set_title(f"{result['oracle']}\n({result['classical_type']})")

axes[i].set_ylabel('Probability')

axes[i].set_xticks(x)

axes[i].set_xticklabels(states)

axes[i].set_ylim(0, 1.1)

axes[i].grid(True, alpha=0.3)

axes[i].legend()

plt.tight_layout()

plt.suptitle('Deutsch-Jozsa Algorithm Results\nComparison of PennyLane and Qiskit Implementations',

y=1.02, fontsize=14)

plt.show()

print("\n" + "="*50)

print("CONCLUSION")

print("="*50)

print("Algorithm Performance Summary:")

print("-" * 40)

correct_count = 0

for result in results:

correct = (result['pennyLane_result'] == result['classical_type'] and

result['qiskit_result'] == result['classical_type'])

if correct:

correct_count += 1

status = "✓" if correct else "✗"

print(f"{result['oracle']:15} {status} {result['classical_type']:9} → "

f"PL: {result['pennyLane_result']:9}, QK: {result['qiskit_result']:9}")

print("-" * 40)

print(f"Overall Accuracy: {correct_count}/{len(results)} ({correct_count/len(results)*100:.1f}%)")

print("\nKey Findings:")

print("1. Both frameworks produce identical results")

print("2. Constant oracles always return |00⟩ with probability 1.0")

print("3. Balanced oracles return other states with probability 1.0")

print("4. Quantum advantage: 1 query vs 3 classical queries")

print("5. Demonstrates exponential speedup for oracle problems")

print("\nMathematical Significance:")

print("- Quantum parallelism evaluates all inputs simultaneously")

print("- Quantum interference reveals global function properties")

print("- Single query determines constant vs balanced classification")

print("- Foundation for more complex quantum algorithms (Grover,